LS-1380L Vertical Machining Center with Dual Tool Magazine Product Specifications

# 1. Basic Information

- Equipment model: LS-1380L vertical machining center with dual tool magazine

- Equipment type: precision CNC vertical machining center

- Application scope: Various metal parts (aluminum alloy, stainless steel, carbon steel, copper alloy, etc.) milling, drilling, boring, tapping, rigid tapping and other multi-process compound processing, widely used in automotive parts, communication accessories, molds, medical devices, aerospace parts and other fields.

- Dimensions (length × width × height): 3200mm × 2800mm × 3000mm

- Device weight: 8500kg

- Installation method: fixed foundation installation, horizontal adjustment

# II. Core Structural Parameters

## 1. Workbench Specifications

- Workbench dimensions (length × width): 1500mm × 800mm

- Maximum workbench load: 1200kg

- Number/Specifications of T-shaped grooves: 5, groove width 18mm, groove spacing 150mm

- Maximum distance from the workbench to the spindle end face: 120-800mm

## 2. Trip Parameters

- X-axis travel (left/right):

- Y-axis stroke (front to back): 800mm

- Z-axis travel (up and down): 700mm

- X/Y/Z axis positioning accuracy: ±0.005mm/1000mm

- X/Y/Z axis repeat positioning accuracy: ±0.003mm

# 3. Spindle System Parameters

- Spindle taper hole: BT40

- Spindle speed range: 100-8000rpm (stepless speed adjustment)

- Spindle power: 11kW (rated) / 15kW (peak)

- Spindle torque: 78N·m (rated) / 105N·m (peak)

- The cooling mode of spindle: forced cooling by oil cooler

- Spindle motor type: servo spindle motor

- Rigid tapping range: M3-M30

# 4. Dual Knife Library Configuration Parameters

## 1. Surgeon's Kit

- Tool magazine type: Chain tool magazine

- 24 knives

- Maximum tool diameter: 100mm (without adjacent tool) / 50mm (with adjacent tool)

- Maximum tool length: 300mm

- Maximum tool weight: 8kg

- Blade change time (blade-to-blade): 1.2s

## 2. Auxiliary Tool Magazine

- Tool magazine type: Bamboo hat type

- 16 knives

- Maximum tool diameter: 80mm (without adjacent tools) / 45mm (with adjacent tools)

- Maximum tool length: 280mm

- Maximum tool weight: 6kg

- Blade change time (blade-to-blade): 0.8s

## 3. Dual knife magazine switching method

- Switch Control: Automatic Switching of CNC System

- Time: ≤5s

- Tool change method: Cam manipulator tool change, precise positioning

# 5. Feed system parameters

- X/Y/Z-axis rapid movement speed: 36m/min, 36m/min, 30m/min

- X/Y/Z-axis feed speed range: 1-10000mm/min

- Feed motor power: 4kW for X/Y axes, 5.5kW for Z axis

- Transmission method: ball screw transmission, pre-stretching treatment

- Guide type: Heavy-duty linear guide, high rigidity, low friction

# VI. CNC system and operating parameters

- Numerical Control System: FANUC 0i-MF or SIEMENS 828D (optional)

- Display screen: 10.4-inch color touchscreen with Chinese/English switching

- Operation modes: manual, automatic, MDI, edit, and handwheel pulse

- Program storage capacity: 10,000 program segments, supports USB flash drive or network transfer

- Accessibility: Tool life management, workpiece coordinate system setup, error compensation (geometric/thermal)

# 7. Hydraulic and pneumatic system parameters

- Hydraulic system pressure: 4.0-5.0MPa

- Hydraulic oil tank capacity: 80L

- Pneumatic system pressure: 0.6-0.8MPa

- Pneumatic air consumption: 150L/min

- Cooling system: 200L coolant tank, forced cooling cycle, and optional constant temperature control

# VIII. Safety and Protection Parameters

- Protective structure: fully enclosed protective cover, oil, water and dust resistant

- Safety devices: emergency stop button, travel limit switch, spindle overload protection, tool magazine safety lock

- Environmental compliance: Meets GB/T 18400.1-2015 mechanical safety standards with noise ≤75dB(A)

# 9. Power and environmental parameters

- Power input: 380V/50Hz/Three-phase four-wire system

- Total installed power: ≤45kW

- Work environment temperature: 5-40℃

- Work environment humidity: 40%-80% (no condensation)

- Altitude: ≤1000m (requires special configuration if exceeded)

# 10. Optional configuration parameters

- Numerical Control (NC) Systems: High-end models such as the FANUC 31i-B

- Detection device: workpiece probe, tool probe

- Chip removal system: screw chip remover + magnetic separator

- Spindle upgrade: 12,000 rpm high-speed spindle, BT50 spindle

- Cooling System: Oil Mist Collector and Constant Temperature Cooling System

- Other: oil-water separator, LED indicator light, machine tool shim

Note: The above parameters are standard configuration, which can be customized according to the actual processing needs of customers. The specific technical agreement signed by both parties shall prevail.

(Note: Some content in this document may be generated by AI.)